| MD191 – MONOPOL | Station: MONO 9 |
| --- | --- |

STATION SUMMARY

Date and time approaching site: 6/6/2012 –6:08 (TU)

Date and time ending station activity:6:6/2012 – 16:35 (TU)

Latitude:16°14.54N		Longitude: 87°54.70E	Water depth (m):  2610

Short area description:Sedimentary levee with a few strong 3.5 kHz reflectors. CASQ core revealed numerous silty layers (turbidites), from 1-2 cm up to 20 cm thick. This stiff layers probably explained the relatively short penetration of the CASQ (only about 6m). Decision to abort CALYPSO coring operations..

| Activity on station |  | Core identification |
| --- | --- | --- |
| Calypso corer | No |  |
| Casq | Yes | MD12-3419-CQ |
| Multi-corer | Yes | MD191-MC-MONO9 |
| Temp/Salinity/Fluorometry on coring device | No |  |
| CTD / Water sampling (rosette) | Yes |  |
| Deep In situ pump | No |  |
| Plankton net | Yes |  |

Figure: 3.5 kHz profile across station MONO 9 while arriving on site.

| MD191 – MONOPOL | Station: MONO 9 |
| --- | --- |

CASQ CORING OPERATION: MD12-3419-CQ

| Coring site information | Latitude (°N): | 16°14.63N |
| --- | --- | --- |
|  | Longitude (°E): | 87°54.67E |
|  | Water depth (m): | 2614 |
| Casq corer settings | Core length (m): | 9 |
|  | Empty weight in water (t): | 6.83 |
| Coring operation | Time corer starts descent: | 13:49 TU |
|  | Time on bottom: | 14:39 TU |
|  | Maximum tension (t) | 11.2 |
|  | Time corer on deck: | 16:00TU |
| Result coring operation | Total weight in water (t) | 7.25 |
|  | Length of sediment (m) | 6.85 |

General observations / incidents:

Nombreux niveaux de siets assez compacts (turbidites) dont un premier niveau vers 3m qui fait 15-20cm.
Le CASQ 9m ne s’est planté que de 6m => suspicion d’un niveau compact à la base et peut-être CQ freiné par le nombre de turbidites.

=> Ce sédiment semble peu propice à du carottage long. On ne peut pas y faire les tests du CALYPSO avec tubes 42m… et encore moins carotter 60m.

=> ABANDON du site pour aller plus au sud sur levée avec épaisse couche fine (3.5 kHz).

MULTI-CORER OPERATION: MD191-MC-MONO9

| Coring site information | Latitude (°N): | 16°14.63N |
| --- | --- | --- |
|  | Longitude (°E): | 87°54.66E |
|  | Water depth (m): | 2608 |
| Coring operation | Time multi corer starts descent: | 10:58 TU |
|  | Time at bottom: | 11:48 TU |
|  | Time corer on deck: | 12:48 TU |
| Length of sediments (cm) | Tube 1 | 42 |
|  | Tube 2 | 44 |
|  | Tube 3 | 42 |
|  | Tube 4 | ~42 |

General observations / incidents:

4 tubes pleins : magnifique !

Echantillonnage de l’eauinterstitielle (PW) a réussi.

Echantillonnage de l’eau de surface a réussi.

Pour la première fois on a récupéré les 4 tubes identiques, bonnequalité de sédiment.

| MD191 – MONOPOL | Station:MONO 9 |
| --- | --- |

CTD/ROSETTE/ISP OPERATION

| CTD/Rosette site information | Latitude (°N): | 16°14.57’N |
| --- | --- | --- |
|  | Longitude (°E): | 87°54.69’E |
| CTD/Rosette operation | Time CTD/rosette starts descent: | 6:33 TU |
|  | Time on bottom: | ? |
|  | Time on deck: | 8:30 TU |
| Depth of Water sampling | Bottle 1 | 2592 |
|  | Bottle 2 | 2592 |
|  | Bottle 3 | 2572 |
|  | Bottle 4 | 2500 |
|  | Bottle 5 | 2200 |
|  | Bottle 6 | 1500 |
|  | Bottle 7 | 1500 |
|  | Bottle 8 | 900 |
|  | Bottle 9 | - |
|  | Bottle 10 | 500 |
|  | Bottle 11 | 300 |
|  | Bottle 12 | 180 |
|  | Bottle 13 | - |
|  | Bottle 14 | 160 |
|  | Bottle 15 | 140 |
|  | Bottle 16 | 110 |
|  | Bottle 17 | 90 |
|  | Bottle 18 | 70 |
|  | Bottle 19 | 50 |
|  | Bottle 20 | 40 |
|  | Bottle 21 | 30 |
|  | Bottle 22 | 20 |
|  | Bottle 23 | 10 |
|  | Bottle 24 | 5 |

MULTI-NET OPERATION

|  |  | First deployement | Second deployement |
| --- | --- | --- | --- |
| Coring site information | Latitude (°N): | 16.24° N | 16.24° N |
|  | Longitude (°E): | 87.91° E | 87.91° E |
| Coring operation | Time start operation: | 9:24 | 10:03 |
|  | Time net on deck: | 9:41 | 10:10 |
| Depth of multinet collection (m) | Collector 1 | 400-180 | 100-80 |
|  | Collector 2 | 180-80 | 80-60 |
|  | Collector 3 | 80-60 | 60-40 |
|  | Collector 4 | 60-40 | 40-20 |
|  | Collector 5 | 40-0 | 20-0 |

General observations / incidents:

| MD191 – MONOPOL | Station: MONO 9 |
| --- | --- |

CTD PROFILES

Figure: Temperature, salinity, fluorescence and dissolved oxygen profiles at site MONO 9.

| MD191 – MONOPOL | Station: MONO 9 |
| --- | --- |

SECTIONS INFORMATIONS OF CASQ MD12-3419-CQ

Number of layers (2 or 3):

Sketch of the core section, defining the location of liners A, B, C, D, E :

B

A

C

E

D

Liner A, D : large size

Liners B, E : medium size

Liners C : small U-channel

SECTIONS INFORMATIONS OF CASQ MD12-3419 CQ

| Core MD12-3419 CQ |  |  |  |  |  |  | MONOPOL -MD191 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Section | Top depth (cm) | Bottom depth (cm) | Real length of section (cm) | Observations |  |  |  |
| I | 0 | 150 | 148.9 | - |  |  |  |
| II | 150 | 300 | 149.3 | - |  |  |  |
| III | 300 | 450 | 150.3 | - |  |  |  |
| IV | 450 | 600 | 150.6 | - |  |  |  |
| V | 600 | 750 | 149.1 | - |  |  |  |
| VI | 750 | 836 | ?? | - |  |  |  |

| MD191 – MONOPOL | Station: MONO 9 |
| --- | --- |

PICTURES OF CASQ MD12-3419-CQ

| MD191 – MONOPOL | Station: MONO 9 |
| --- | --- |

PHYSICAL PROPERTIES OF CASQ MD12-3419 -CQ

Figure: Color reflectance and physical properties of CASQ core MD12-3419 CQ

| MD191 – MONOPOL | Station: MONO 9 |
| --- | --- |

SEDIMENTOLOGY OF CASQ CORE MD12-3419-CQ